3GPP TSG-SA WG6 Meeting #70	S6-255642

Dallas, United States 17th – 21st November 2025	(revision of S6-255322, S6-255122)

Title:	Follow-up reply LS on N6 measurement exposure

Response to:	Reply LS on N6 measurement exposure (S2-2509811/S6-255010)

Release:	3GPP Rel-20

Work Item:	TEI20, EDGEAPP_Ph3

Source:	3GPP TSG SA WG6

To:	3GPP TSG SA WG2

Cc:	n/a

Contact person:	Walter Featherstone

w_featherstone@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	n/a

# 1	Overall description

SA6 thanks SA2 for their Reply LS on N6 delay measurement exposure and provides the following response:

Regarding “SA2 requires further details to decide whether and how to support N6 delay measurement exposure to SA6-defined entity (considered as AF from SA2 perspective) in Release 20. SA2 kindly asks SA6 to provide further details, e.g., scenarios, for N6 delay measurement exposure.”

SA6 acknowledges the use of N6 delay measurement by the SMF for Local UPF (re)selection. SA6’s intent is to consume these network-sourced measurements in a complementary manner at the application enablement layer, without duplicating SMF functions.

Specifically, for application enablement layer specific mechanisms (e.g., candidate ranking in EAS discovery (re)selection, supporting application context relocation timing/target decisions, EEC notifications conveying the current suitability of the selected EAS) and for requesting AF traffic influence toward the selected EAS. This is without requesting or implying L-UPF (re)selection, which remains under SMF control.

Regarding “SA2 has concern on potential network topology information exposure in case where UPF related information is paired with N6 delay measurement information exposure.”

To leverage N6 delay measurement information at the enabler layer, an SA6-defined entity (e.g., an Edge Enabler Server acting as AF) needs to be able to distinguish the N6 delay measurement per EAS measurement endpoint in its DN (reference 3GPP TS 23.501 clause 5.8.2.23).

Based on the above additional information and use-case clarification, SA6 would like to ask SA2 whether they could support exposure of N6 delay measurement information to SA6-defined entities that is unique per N6 measurement endpoint, whilst not revealing network topology information.

# 2	Actions

To SA2 (ACTION):

SA6 kindly requests SA2 to consider support for exposure of N6 measurement information that is unique per measurement endpoint.

# 3	Dates of next TSG SA WG 6 meetings

SA6#71                 9th – 13th February 2026 		Goa, India

SA6#72                 13th – 17th April 2025 		Malta